- Interplay of inversion-less and inversion-based terahertz stimulated intracenter emission

- in isotopically enriched silicon doped by bismuth

- S.G. Pavlov 1), N.V. Abrosimov 2), N. Deßmann 3), U. Böttger 1), B. Redlich 4), A.F.G. van der Meer 4), R.Kh. Zhukavin 5), V.N. Shastin 5), K. Irmscher 2), H. Riemann 2) and H.-W. Hübers 1,3)

- 1) Institute of Planetary Research, German Aerospace Center, Berlin, Germany

- 2) Leibniz Institute of Crystal Growth, Berlin, Germany

- 3) Institut für Optik und Atomäre Physik, Technische Universität Berlin, Berlin, Germany

- 4) FELIX Facility, Radboud University, Nijmegen, The Netherlands

- 5) Institute for Physics of Semiconductors, Nizhny Novgorod, Russia

Two types of terahertz-range stimulated emission involving intracenter transitions of bismuth atoms in isotopically enriched silicon has been realized under optical excitation by mid-infrared free electron laser (FELIX Facility, Radboud, the Netherlands) at cryogenic temperatures, ~ 5 K. In comparison with the similar stimulated emission observed for natural silicon crystals [1], strong interplay between Raman laser emission and inverted intracenter lasing has been observed (Fig. 1). This competition is caused by interaction of particular donor levels those populations play a critical role in forming inverted electron distributions required for intracenter laser action (for instance 2p 1s(E) bismuth transition in Fig. 1) versus dominant population of the ground impurity state (inversion-less), 1s(A1), what is required for Raman Stokes lasing.

Fig. 1. Left: 28Si:Bi lasing scheme showing pump (arrow upwards) and emission transitions (arrows downwards). Right: Dynamics of the laser emission pulses (lower) and frequency (upper) during the FEL pump pulse.

Authors acknowledge the support through the EU initiative “CALIPSO” (Project # FELIX-2013-2-2) and the Russian Foundation for Basic Research (grants # 14-02-00638, 13-02-97116-povolzhie).

References

- S. G. Pavlov, U. Böttger, R. Eichholz, N. V. Abrosimov, H. Riemann, V. N. Shastin, B. Redlich, and H.-W. Hübers. Terahertz lasing from silicon by infrared Raman scattering on bismuth centers. // Appl. Phys. Lett. 95, 2009 р. 201110.